Cs 355 Computer Architecture

Processor: Datapath & Control

Text: Computer Organization & Design, Patterson & Hennessy

Chapter 4-4.4

Objectives:  The Student shall be able to:

- Define the use of the following: IR, PC, register file, ALUout, multiplexer, instruction cache, data cache, control signals.

- Deduce the operations that are necessary to complete any specified instruction.

- Define and describe single-cycle and multi-cycle data path.

- Describe what happens in an exception or interrupt.

Class Time:

Lecture					1.5 hours

Lab						1.5 hours

Total                                                                3 hours

# Components of the Processor

PC (Program Counter): Holds the address of the next instruction

IR (Instruction Register): Holds the executing instruction

Instruction Cache: ‘Fast’ memory where the next instruction comes from

Reg[index] (Register File): Contains the 32 registers

Arithmetic Logic Unit (ALU): Performs all arithmetic operations

Data Cache:  Data read from or written to ‘fast’ memory

Control:  Tells devices when to do their thing and what to do

- The op-code and function code define which control bits are raised.

- The edge of the clock cycle is when operations are initiated, registers are read.

Multiplexer:  Multiple inputs selects one output based upon control signal(s)

- E.g. to the Register File:  Are we reading from rt, rd or 31?  RegDst control signals indicate

- E.g. to the ALU: Are we reading from the Instruction (Imm) or a register? ALUSrc control signals indicate.

- E.g. to Data out:  send data from data cache, ALU, or next PC to register file?  RegInSrc control signals indicate

Clock cycle:

Rising edge

Single-Cycle Data Path:  Each instruction executes in one clock cycle

Multi-Cycle Data Path:  Each instruction takes multiple clock cycles

R/I/J instruction formats:

| Opcode 6 bits | RS 5 bits | RT 5 bits | RD 5 bits | Shift amt 5 bits | Function Code 6 bits |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Immediate Operand 32 bits |  |  |
|  | Jump Target Address 26 bits |  |  |  |  |

### Operations

Instruction Fetch Step

// Read into Instruction Register from Memory at address indicated by Program Counter register.  This is always done, since we don’t know what type of instruction it is yet.

IR <= InstrCache[PC];

PC <= PC + 4;		// Increment Program Counter to point to next instruction

Assume this step takes 2 ns.

Instruction Decode and Register Read Step

// This is always done, since we don’t know what type of instruction it is yet.

RS <= RegFile[IR[25:21]];

RT <= RegFile[IR[20:16]];

Immediate <= sign-extend(IR[15:0]);

pcALUOut <= PC + (Immediate << 2);
 Assume this step takes 1 ns.

ALU Operation:  This is instruction-specific, since control knows the instruction type.

R-Type Instruction: ALUOut <= RS op RT

Memory-Reference Instr:  ALUOut <= RS + Immediate

Branch Instruction:  If (RS compare RT) PC <= pcALUOut

Jump Instruction:  PC <= (PC(31:28), (IR[25:0],2*b00))

Assume this step takes 2 ns.

Completion 1: Data Cache Access

R-Type Instruction:  RegFile[IR[15:11]] <= ALUOut

(This instruction may occur in next step instead, depending on design)

Memory-Reference Instr:

Load Instr: DataCacheOut <= DataCache[ALUOut]

Store Instr: DataCache[ALUOut] <=RT

Assume this step takes 2 ns.

Completion 2: Register Write-Back

Load Instr: RegFile[IR[20:16]] <= DataCacheOut

Assume this step takes 1 ns.

Questions to consider:

- How long will an R-Type instruction take?

- How long will a Branch or Jump instruction take?

- How long will a Store instruction take?

- How long will a Load instruction take?

- With a single-cycle data path, how long should an instruction cycle be?

Single-Cycle Data Path:

- Every cycle must be equal length

- The cycle time must be long enough to accommodate the longest instruction.

Solution:  Multi-cycle Data Path

- Have the cycle time coincide with the instruction stage time.

- Cycle time = 2 ns (time of longest stage)

- How long does it take to execute a Branch, Load, Store, Register instruction now?

- Between stages we need registers to hold data for next stage.

### Exceptions

Exceptions can include:

- Integer or floating point overflow or underflow

- Invalid op-code

- Sensor indicates fault condition (e.g., overheating)

- Cache miss (data not in cache – must go to memory)

Actions taken with exception:

- Save reason in the Cause register

- Save address of current instruction in the Exception Program Counter (EPC) register

- Transfer control to address SysCallAddr

- Execute exception state logic

## Exercise

Design the instructions:

addi     $s1, $s2, 24

slt    $t1,$s1,$s2

lw    $s1,24($s2)

bne   $zero,$t3,loop

sw  $s3,25($s4)

jal  target